ASSIGNMENT 02

KANWAL SHEHZADI-------------------------------------------------02-131212-027

ASSIGNMENT 02

- Write a program to display a well come to bahria in LCD display

With Arduino:

C PROGRAM:

CIRCUIT:

With 8051:

C CODE:

#include <htc.h>  // Include the header file for your microcontroller (e.g., 8051)

void delay(unsigned int time) {

unsigned int i, j;

for(i = 0; i < time; i++)

for(j = 0; j < 1275; j++);

}

void lcd_command(unsigned char command) {

P20 = 0;   // Select command register

P21 = 0;   // Set to write mode

P1 = command;  // Send the command to data pins

P22 = 1;   // Enable the LCD

delay(1);

P22 = 0;   // Disable the LCD

}

void lcd_data(unsigned char dat) {

P20 = 1;   // Select data register

P21 = 0;   // Set to write mode

P1 = dat;  // Send the data to data pins

P22 = 1;   // Enable the LCD

delay(1);

P22 = 0;   // Disable the LCD

}

void lcd_init() {

lcd_command(0x38);  // 8-bit, 2-line display, 5x7 font

lcd_command(0x0E);  // Display on, cursor blinking

lcd_command(0x01);  // Clear display

lcd_command(0x06);  // Increment cursor

lcd_command(0x80);  // Set cursor to the first line

}

void main() {

lcd_init();

lcd_data('W');

lcd_data('e');

lcd_data('l');

lcd_data('c');

lcd_data('o');

lcd_data('m');

lcd_data('e');

lcd_data(' ');

lcd_data('t');

lcd_data('o');

lcd_data(' ');

lcd_data('B');

lcd_data('a');

lcd_data('h');

lcd_data('r');

lcd_data('i');

lcd_data('a');

while (1);

}

ASSEMBLY PROGRAM:

#include <htc.h>  ; Include the header file for your microcontroller (e.g., 8051)

delay_ms MACRO delay

mov R2, #delay

mov R3, #20

DLY_LOOP:

DJNZ R3, DLY_LOOP

DJNZ R2, DLY_LOOP

ENDM

delay_us MACRO delay

mov R2, #delay

DLY_LOOP_US:

NOP

DJNZ R2, DLY_LOOP_US

ENDM

lcd_command MACRO command

CLR P2.0     ; Select command register

CLR P2.1     ; Set to write mode

MOV A, command

MOV P1, A    ; Send the command to data pins

SETB P2.2    ; Enable the LCD

delay_us 1

CLR P2.2     ; Disable the LCD

ENDM

lcd_data MACRO data

SETB P2.0    ; Select data register

CLR P2.1     ; Set to write mode

MOV A, data

MOV P1, A    ; Send the data to data pins

SETB P2.2    ; Enable the LCD

delay_us 1

CLR P2.2     ; Disable the LCD

ENDM

lcd_init MACRO

lcd_command 0x38  ; 8-bit, 2-line display, 5x7 font

lcd_command 0x0E  ; Display on, cursor blinking

lcd_command 0x01  ; Clear display

lcd_command 0x06  ; Increment cursor

lcd_command 0x80  ; Set cursor to the first line

ENDM

ORG 0x0000

LJMP MAIN

MAIN:

MOV A, #0x20  ; Initialize the LCD data pins

MOV P1, A

ACALL lcd_init

ACALL lcd_data, 'W'

ACALL lcd_data, 'e'

ACALL lcd_data, 'l'

ACALL lcd_data, 'c'

ACALL lcd_data, 'o'

ACALL lcd_data, 'm'

ACALL lcd_data, 'e'

ACALL lcd_data, ' '

ACALL lcd_data, 't'

ACALL lcd_data, 'o'

ACALL lcd_data, ' '

ACALL lcd_data, 'B'

ACALL lcd_data, 'a'

ACALL lcd_data, 'h'

ACALL lcd_data, 'r'

ACALL lcd_data, 'i'

ACALL lcd_data, 'a'

MAIN_LOOP:

SJMP MAIN_LOOP

- Draw the Block diagram of seven segment display and write its code in C and Assembly

C PROGRAM:

#include<reg51.h>

void msdelay(unsigned int time)  // Function for creating delay in milliseconds.

{

unsigned i,j ;

for(i=0;i<time;i++)

for(j=0;j<1275;j++);

}

void main()

{

unsigned char to_disp[]={0x3F,0x06,0x5B,0x4F,0x66,        // Array for hex values (0-F) for

0x6D,0x7D,0x07,0x7F,0x6F}        // common anode 7 segment

int k;

while(1)

{

for(k=0;k<10;k++)

{

P2=to_disp[k];

msdelay(100);

}

}

}

ASSEMBLY PROGRAM:

ORG 4000H

DB 3FH, 06H, 5BH, 4FH, 66H, 6DH, 7DH, 07H, 7FH, 6FH, 0

; Lookup table for digits 0 to 9

ORG 0000H

main:   MOV DPTR, #4000H

repeat: CLR A

MOVC A, @A+DPTR       ; Copy data from external location to accumulator

MOV P2, A             ; Move the pattern of the digit into port P2

ACALL delay           ; Call a delay to so that the transition is visible

INC DPTR              ; Point to the next pattern

CJNE A, 0, repeat     ; Repeat till 0 (Stop bit) is received

SJMP main             ; Run this forever till externally stopped

; generate a decent enough delay between transitions

delay:

MOV R0, #08H

LP2:    MOV R1, #0FFH

LP1:    MOV R2, #0FFH

LP3:    DJNZ R2, LP3

DJNZ R1, LP1

DJNZ R0, LP2

RET

END

CIRCUIT:

- Draw block diagram of two seven segment display

Write a program in Assembly and C to display a digit from 0 to 9 in both seven segment display

C PROGRAM:

#include<reg51.h>

void delay(unsigned int t);

void main()

{

unsigned int ch[]={0xC0,0xF9,0xA4,0xB0,0x99,0x92,0x82,0xF8,0x80,0x90};

unsigned int i,j;

P3=0xc0;

P2=0xc0;

while(1)

{

for(j=0;j<10;j++)

{

for(i=0;i<10;i++)

{

P3=ch[i];

delay(50);

}

if(j!=10)

P2=ch[j+1];

}

if(i==10&&j==10)

P3=0xc0;

P2=0xc0;

}

}

void delay(unsigned int t)

{

unsigned int i,j;

for(i=0;i<t;i++)

for(j=0;j<1275;j++);

}

ASSEMBLY PROGRAM:

ORG 0000H

MOV DPTR, #patterns   ; Initialize DPTR with the starting address of the patterns

MOV R2, #0Ah          ; R2 = 10 (to iterate 10 times)

display_loop:

MOV A, R2          ; Move the count in R2 to the accumulator

MOVX A, @DPTR      ; Move the data from the address pointed by DPTR to the accumulator

MOV P3, A          ; Display the value from the array on port P3

ACALL delay        ; Call delay function to create a delay

INC DPTR           ; Point to the next pattern in the array

DJNZ R2, display_loop  ; Repeat until R2 becomes 0

MOV P3, #0xC0      ; Reset P3 to initial value 0xC0

MOV P2, #0xC0      ; Reset P2 to initial value 0xC0

endless_loop:

SJMP endless_loop  ; Infinite loop

delay:                 ; Delay function definition

MOV R7, #5         ; Delay count outer loop

outer_loop:

MOV R6, #50        ; Delay count inner loop

inner_loop:

NOP                ; No operation, to increase delay

DJNZ R6, inner_loop

DJNZ R7, outer_loop

RET

patterns:              ; Define patterns array

DB 0xC0, 0xF9, 0xA4, 0xB0, 0x99, 0x92, 0x82, 0xF8, 0x80, 0x90

DB 0             ; Stop bit

END

CIRCUIT DIAGRAM: